MAPLE-SYRUP

CLI User Guide

Installation, input preparation, CPU and GPU runs, restart, outputs, plotting and the complete command reference.

Sediment Yield, Runoff, and Uptake by Plants

Model version   SYRUP d05d8c6

Qualified MAPLE   39bcf79

Prepared   20 October 2026

Document type   Practical reference

The acronym names future plant uptake; growth and nutrient/ecohydrological feedbacks are outside the implemented water-model scope.

# Contents

1. Install MAPLE and SYRUP

2. Check the installation: the probe

3. What you must supply: MAHLERAN inputs (not in Git)

4. Directory layout you will end up with

5. Commands: nine installed console scripts, no single syrup command

6. Plot 1 end-to-end

7. Event semantics you need to interpret results

8. Other workflows

9. Outputs: files, units, orientation

10. Figures and movies

11. Scope limits

12. Troubleshooting

13. Reproducibility checklist

Appendix A. Complete CLI option reference

Appendix B. References and verification scope

This guide is for a new scientific collaborator. It shows how to install the exact pinned sources, import the corrected MAHLERAN Plot 1 case, run the conservative water–sediment event on CPU and GPU, pause and resume, and read the outputs. Each section connects commands with their intended workflow and outputs.

Pinned sources used throughout:

| Component | Repository | Exact commit |
| --- | --- | --- |
| MAPLE (wind/bed model, dependency) | https://github.com/gregokin/MAPLE.git | 39bcf79e1e67b6ed960f78a923ddfedf1d173e68 |
| SYRUP (maple-syrup, this package) | https://github.com/gregokin/SYRUP.git | d05d8c6c66d9fd2969c007681916254e8d1c2c7c |
| MAPLE package digest the storage guards accept | — | af54626ab432670539ed71480fd884f13581dfdfcfec63d7c85053ca215adfec |

Both projects need Python ≥ 3.11. The qualified interpreter was CPython 3.12.3 with NumPy 2.5.3, Numba 0.67.0 / llvmlite 0.49.0 and, for GPU, CuPy 14.2.0 with CUDA 12.9 toolkit components.

An auto-generated reference appendix covering every flag, default and choice is appended after this guide. The guide itself explains how the important controls fit together and which combinations are refused.

Companion materials. This guide, its source Markdown and the companion plotting example render_outputs.py are distributed together as SYRUP_Manuals_and_Examples.zip. They are not part of the pinned SYRUP commit d05d8c6, so a fresh clone does not contain them. Unpack the archive to $WORK/manuals (Section 1.1); every plotting command below runs the helper from there.

# 1. Install MAPLE and SYRUP

## 1.0 System prerequisites

The commands below use Bash on Linux or WSL2. Run them in that environment; native PowerShell syntax and paths differ.

You need:

Git;

Python ≥ 3.11 (3.12 qualified) with the venv module and pip;

optionally, FFmpeg, only to turn PNG frames into a video;

for GPU runs only, an NVIDIA driver supporting CUDA 12.x, installed on the host (on WSL2, the Windows driver). The driver is not a pip package.

On Ubuntu 24.04, whose system Python is 3.12, the system packages are typically:

sudo apt update
sudo apt install git python3.12 python3.12-venv python3-pip ffmpeg   # ffmpeg optional

These commands are an example of what to install, not a required recipe. Everything else (the MAPLE base dependencies, Numba, CuPy, packaging, Matplotlib) is installed into the virtual environments with pip below.

## 1.1 Choose absolute paths once

Every later command uses these variables. Pick a directory you own; nothing below writes outside it except pip's own caches.

export WORK=$HOME/syrup-work                 # any absolute path you own
export MAPLE=$WORK/MAPLE-39bcf79             # NEW, separate MAPLE checkout
export SYRUP=$WORK/SYRUP-d05d8c6             # SYRUP checkout
export MAHLERAN=$WORK/MAHLERAN               # your copy of the MAHLERAN inputs (Section 3)
export RUNS=$WORK/runs                       # cases, outputs, checkpoints, stores
export MANUALS=$WORK/manuals                 # unpacked SYRUP_Manuals_and_Examples.zip
export BIN=$MAPLE/.venv/bin                  # CPU environment
export GBIN=$MAPLE/.venv-cupy/bin            # optional GPU environment
export PYTHONDONTWRITEBYTECODE=1
mkdir -p "$WORK" "$RUNS"

Keep $MAPLE and $SYRUP outside $RUNS. The runners refuse to write into the MAPLE tree, the MAHLERAN tree, the recipe directory or the SYRUP package.

## 1.2 Clone MAPLE at the exact commit

Use a new clone. If you already have a MAPLE checkout that you use for other work, do not repoint it: shared checkouts move forward, and SYRUP binds runs to the exact MAPLE source digest.

cd "$WORK"
git clone https://github.com/gregokin/MAPLE.git "$MAPLE"
cd "$MAPLE"
git checkout --detach 39bcf79e1e67b6ed960f78a923ddfedf1d173e68
git rev-parse HEAD        # must print 39bcf79e1e67b6ed960f78a923ddfedf1d173e68
git status --porcelain    # should print nothing before you install

## 1.3 Create the CPU environment inside the MAPLE clone

cd "$MAPLE"
python3.12 -m venv .venv
"$BIN/python" -m pip install --upgrade pip
"$BIN/python" -m pip install -e "$MAPLE[windowed]"

This installs MAPLE as an editable distribution named maple, plus its base dependencies from MAPLE's pyproject.toml: jax, jaxlib, numpy, pyyaml, xarray>=2024.10, zarr>=3.0, dask[array] and tifffile. The windowed extra adds numba>=0.67,<0.68. Without Numba, MAPLE's exact CPU height decode in the resident-window bed refuses with PrefixPacketError: decode_height on NumPy arrays needs the compiled Numba kernel ....

Never run `pip install maple` from PyPI. The PyPI project named maple is unrelated. MAPLE is not published, so SYRUP deliberately does not declare it as a dependency. Instead, at runtime SYRUP checks which maple was imported (maple_syrup.dependency) and refuses anything it cannot identify.

## 1.4 Optional: create the GPU environment

cd "$MAPLE"
python3.12 -m venv .venv-cupy
"$GBIN/python" -m pip install --upgrade pip
"$GBIN/python" -m pip install -e "$MAPLE[windowed,gpu]"

The gpu extra is cupy-cuda12x[ctk]>=14.0,<15. It is CuPy, not GPU JAX. The [ctk] part pulls in the CUDA 12 toolkit headers that CuPy needs to JIT-compile kernels. Without them, the first kernel launch fails with Failed to find CUDA headers. CUDA 12 was chosen because it still supports Pascal (SM 6.1, e.g. GTX 1080 Ti); CUDA 13 does not. The qualified point is CuPy 14.2.0 with CUDA 12.9 components. Other versions in the range are allowed but untested.

The NVIDIA driver is not a pip package. Install it separately (on WSL2 that is the Windows host driver). Check with nvidia-smi that it supports CUDA 12.x.

## 1.5 Clone SYRUP and install it into the same environment(s)

cd "$WORK"
git clone https://github.com/gregokin/SYRUP.git "$SYRUP"
cd "$SYRUP"
git checkout --detach d05d8c6c66d9fd2969c007681916254e8d1c2c7c
git rev-parse HEAD        # must print d05d8c6c66d9fd2969c007681916254e8d1c2c7c

"$BIN/python" -m pip install -e "$SYRUP[numba,dependency-delivery]"
"$BIN/python" -m pip install matplotlib          # only for plotting outputs
# GPU environment, if created:
"$GBIN/python" -m pip install -e "$SYRUP[numba,dependency-delivery]"
"$GBIN/python" -m pip install matplotlib

numba turns on the compiled CPU kernels; nothing falls back silently if they are missing. dependency-delivery (packaging>=23) is used only by the maple_syrup.dependency_bundle helper.

Matplotlib is not a base dependency of either package. MAPLE lists it only in its optional viz extra, together with Panel and hvPlot for MAPLE's own interactive viewer. You do not need MAPLE[viz] to plot SYRUP's NPZ outputs; installing Matplotlib alone is enough for the companion plotting example.

Finally, tell SYRUP which MAPLE it must find:

export MAPLE_SYRUP_EXPECTED_MAPLE_ROOT=$MAPLE

With this set, every resolver call refuses any other MAPLE source root. The plot import, dense event and most other runners also accept --expected-maple-root. The windowed runner reads only the environment variable.

## 1.6 Using an existing machine's shared environments instead

On a machine whose shared MAPLE environments (for example /home/okin/MAPLE/.venv and .venv-cupy) already contain the third-party packages, you can reuse them without creating new venvs. However, their editable maple points at that machine's live checkout, which has moved past 39bcf79. Do not install your new clone or SYRUP into them. Instead, place the verified materialized MAPLE 39bcf79 bundle ahead of site-packages, and run SYRUP from your pinned source with python -m.

A shared environment may already contain maple-syrup-* console scripts from some other installation. Those scripts launch whatever SYRUP that installation points at, not your pinned clone, so do not use them here. Run every command as "$PY" -m maple_syrup.<module> with the PYTHONPATH set below, and confirm the selected sources with check-runtime and the probe.

PY=/home/okin/MAPLE/.venv/bin/python             # or .venv-cupy/bin/python for GPU
BIN=$(dirname "$PY")                           # interpreter for the companion plotting commands
GBIN=/home/okin/MAPLE/.venv-cupy/bin             # shared GPU interpreter, when selected
C=/home/okin/SYRUP/outputs/dependencies/content_cache/37a1bc69bfa034be7cd512d8aa7c30d33682b7c053a53c3b0c91edc34d59a96f
PYTHONPATH=$SYRUP/src "$PY" -m maple_syrup.dependency_bundle env "$C" --syrup-src "$SYRUP/src"   # prints; runs nothing
# export exactly what it prints, typically:
export PYTHONPATH=$C/editable:$C/source/src:$SYRUP/src
export MAPLE_SYRUP_EXPECTED_MAPLE_ROOT=$C/source
export MAPLE=$C/source                          # matches explicit --expected-maple-root in later examples
"$PY" -m maple_syrup.dependency_bundle check-runtime "$C"

The bundle (ID 37a1bc69…) is a local artifact, not something a Git clone provides. If it is missing, use the fresh-clone route in Sections 1.2–1.5. To rebuild such artifacts, see dependencies/maple_shared_storage/README.md and docs/cw_e2_dependency_delivery.md. In that route, substitute "$PY" -m maple_syrup.<module> for every console command below. Also do not run from a directory that contains a stray maple-*.dist-info: the resolver correctly refuses the shadowing distribution.

# 2. Check the installation: the probe

cd "$WORK"
"$BIN/maple-syrup-probe" --expected-maple-root "$MAPLE" --output "$RUNS/probe_cpu.json"
CUDA_VISIBLE_DEVICES=0 "$GBIN/maple-syrup-probe" --backend cupy --expected-maple-root "$MAPLE" \
  --output "$RUNS/probe_gpu.json"        # optional

The probe resolves MAPLE, checks the API surface SYRUP uses, runs a tiny zero-demand MAPLE water exchange and prints a JSON report. --output must name a new file outside the MAPLE tree. Check these fields:

status is "ok";

maple.dependency.source_root is your $MAPLE;

maple.package_source_digest.digest_sha256 is af54626ab432670539ed71480fd884f13581dfdfcfec63d7c85053ca215adfec.

A different digest means the imported MAPLE is not the qualified 39bcf79 source. Opt-in overlay storage will then be refused (Section 6.3).

Exit codes: 0 ok; 1 failed probe check; 2 MAPLE dependency refusal, unavailable backend or refused output file. A CuPy probe only round-trips a tiny state. It is not evidence of GPU readiness.

# 3. What you must supply: MAHLERAN inputs (not in Git)

The model runs on cases imported from MAHLERAN 1.2.x inputs. Neither repository contains these inputs. Obtain them from your collaborators or a MAHLERAN distribution, and treat that tree as read-only.

For corrected Plot 1, cases/plot1/recipe.yaml expects:

$MAHLERAN/
  mahleran_input.xml          # version 1.2.3 (checked); its input_folder must resolve to Input/input_p1
  Input/input_p1/
    p1dem.asc                 # 62 x 22, 0.5 m cells (checked against the recipe)
    p1vegcover.asc  rm_new.asc  p1pavcoverveg.asc  thetasat39.asc
    <surface-type map named in the XML>
    plot1_phi1.asc ... plot1_phi6.asc
    p1_01_08_06.dat           # rainfall series named by the XML (copied verbatim)
    (p1ksat290806.asc, p1sm290905.asc are hashed only if present)

Rules the importer enforces:

every staged raster must share the DEM header;

the DEM must have no nodata cells;

the six grain fractions must close within 1e-6;

no `calib.dat` may exist in Input/input_p1, since legacy calibration multipliers are unsupported.

"Corrected" means the recipe deliberately uses the six distinct plot1_phi1..6.asc maps. The supplied mahleran_input.xml maps all six classes to plot1_phi1.asc; that is a recorded data defect, not a reproduction target. Bed thickness, bulk density (1250 kg m⁻³), voxel size (0.1 m) and active layer (2 mm) are explicit assumptions documented in the recipe, not MAHLERAN values.

# 4. Directory layout you will end up with

$WORK/
├── MAPLE-39bcf79/                 # pinned MAPLE clone (read-only in use)
│   ├── .venv/  .venv-cupy/        # environments (Section 1)
│   ├── pyproject.toml
│   └── src/maple/
├── SYRUP-d05d8c6/                 # pinned SYRUP clone (read-only in use)
│   ├── cases/plot1/recipe.yaml    # authored recipes (input only; never write here)
│   ├── cases/rfid/recipe.yaml
│   ├── cases/chastre/recipe.yaml
│   ├── src/maple_syrup/
│   └── docs/  dependencies/
├── MAHLERAN/                      # external legacy inputs (Section 3), read-only
└── runs/
    ├── cases/plot1_maple39/       # imported + compiled case package
    │   ├── case.yaml  provenance.yaml
    │   ├── source/mahleran_input_p1/*.asc        # staged copies of the inputs
    │   ├── source/syrup_derived/composition_codes.npy
    │   ├── processed/  diagnostics/              # MAPLE compiler artifacts
    │   └── syrup/
    │       ├── plot1_binding.json                # written last; only then is the import complete
    │       ├── plot1_import_report.json  plot1_fields.npz
    │       └── rainfall/p1_01_08_06.dat
    ├── event_cpu/                 # one run's outputs (must not exist beforehand)
    │   ├── completion_summary.json  final_state.npz  hydrograph.npz
    │   ├── handoff/                # dry-handoff bundle (complete runs only)
    │   └── exchange_history.npz    # only with --record-exchange-history
    ├── event_cpu_checkpoints/
    │   └── step_000001234/         # checkpoint.json, maple_state.npz, continuation.npz
    └── stores/win_cpu/             # resident-window deep store (MAPLE-owned layout)

Paths a run creates must be new: the output directory, its sibling <output>_checkpoints (unless you pass --checkpoint-dir to the dense runner) and, for a fresh windowed run, the --store-dir. A run builds its output in a hidden .<name>.partial-* sibling and renames it into place only after it succeeds.

The case package records the absolute recipe path and MAHLERAN root. Later runs re-audit it against those paths. Do not move or edit `$SYRUP/cases/plot1/recipe.yaml` or `$MAHLERAN` after importing.

# 5. Commands: nine installed console scripts, no single syrup command

There is no unified syrup command. The SYRUP pyproject.toml installs nine console scripts. Each one is equivalent to running its module with python -m.

| Console script | python -m module | Purpose | Backends |
| --- | --- | --- | --- |
| maple-syrup-probe | maple_syrup.probe | dependency/API/provenance check | numpy, cupy |
| maple-syrup-plot1-import | maple_syrup.case_import | audit + compile corrected Plot 1 | (CPU) |
| maple-syrup-columns | maple_syrup.column_experiment | no-routing infiltration columns; diagnostic | numpy, cupy |
| maple-syrup-storm | maple_syrup.storm_experiment | water-only storm and recession on frozen terrain | numpy (array/numba), cupy (cuda) |
| maple-syrup-sediment | maple_syrup.sediment_experiment | water + wet sediment over a time window (no stop criterion, reset or restart) | numpy, cupy |
| maple-syrup-event | maple_syrup.event_experiment | conservative complete event, dense MAPLE bed, Plot 1 | numpy (numba/array), cupy (cuda) |
| maple-syrup-windowed | maple_syrup.windowed_experiment | conservative complete event, MAPLE resident-window bed; plot1/rfid/chastre | numpy (numba), cupy (cuda) |
| maple-syrup-benchmark | maple_syrup.benchmark_experiment | matched fixed-terrain benchmark; default dispatches to the legacy replay | numpy only |
| maple-syrup-legacy | maple_syrup.legacy_experiment | MAHLERAN legacy sediment replay, Plot 1 | numpy only |

Module-only user commands (no console script):

| Module | Purpose |
| --- | --- |
| maple_syrup.rfid_case | import RFID_2014 (water benchmark case) |
| maple_syrup.chastre_case | plan / generate / verify tiled Chastre case |
| maple_syrup.experimental_experiment | experimental water-only explicit / local-inertial solvers, with snapshots |
| maple_syrup.legacy_driver | legacy replay driver for plot1 / rfid / chastre, CPU/Numba |
| maple_syrup.legacy_gpu_driver | resident-GPU legacy replay, plot1 / rfid / chastre |
| maple_syrup.dependency_bundle | export / verify / materialize / env / check-runtime of MAPLE bundles |
| maple_syrup.initial_reimport | maintainer-only proposal/import of a generation-zero initial assembly under an independently reviewed exact source transition; not wet checkpoint migration |

## 5.1 Which runner should I use?

| You want | Use |
| --- | --- |
| An actual conservative evolving bed: supply-limited pickup from the real MAPLE active layer and voxels, net-elevation hydraulic refresh, complete event, dry reset, checkpoint/resume | maple-syrup-event (dense bed, Plot 1) or maple-syrup-windowed (resident window; Plot 1, RFID, Chastre) |
| Comparison with the MAHLERAN legacy program | maple-syrup-benchmark / maple-syrup-legacy (default --transport-scheme legacy) or legacy_driver |
| Water only | maple-syrup-storm, experimental_experiment |
| Infiltration diagnostics | maple-syrup-columns |
| Sediment over a fixed window, no completion | maple-syrup-sediment |

The two "mainstem" workflows have different defaults, so they are easy to confuse:

|  | Benchmark default | Conservative event default |
| --- | --- | --- |
| Transport | legacy replay | characteristic phase-bin scheme, --phase-bins 32, dense phase storage (windowed: CSR, --phase-max-nnz 1000000) |
| Sediment supply | fixed composition, unlimited supply, explicit clipping source | MAPLE's native exchange: demand capped by currently *available* mass (the same rule as wind), then by active-layer holdings, with refill from the voxel bed; no clipping source |
| Bed | no evolving MAPLE bed; frozen terrain and routing | evolving bed; reroute only on net elevation change ≥ 0.00075 m |
| Step | fixed dt = 1 s | adaptive, --max-dt-s 1 |
| Completion, dry handoff, restart | none | yes (Section 7) |
| GPU | refused (legacy_gpu_driver is separate) | --backend cupy |

The legacy replay is a benchmark of the old program's behaviour, not MAPLE authority.

Wind coupling. A completed conservative event ends with a dry handoff: the dry native MAPLE bed, which a qualified wrapper can pass to MAPLE's wind model. No SYRUP command invokes a wind event. Sequential wind–water coupling is MASY's job (Section 11). The legacy replay produces no handoff bed at all.

## 5.2 CPU vs GPU: explicit pairings, no fallback

| Runner | CPU | GPU |
| --- | --- | --- |
| maple-syrup-event | --backend numpy --implementation numba (default) or array | --backend cupy --implementation cuda (--cuda-mode auto/fused/split) |
| maple-syrup-windowed | --backend numpy (Numba hydraulics implied) | --backend cupy (CUDA hydraulics implied) |
| maple-syrup-storm / -sediment | --implementation numba (default) or array | --backend cupy --implementation cuda |
| experimental_experiment | --backend numpy --implementation numpy/numba | --backend cupy --implementation cuda |
| maple-syrup-legacy, maple-syrup-benchmark | numpy only | refused; use legacy_gpu_driver |

For these runners, any pairing not listed in the table (for example --backend cupy with --implementation numba, or --implementation cuda on NumPy) is refused before the case is read. Nothing falls back between CPU and GPU. The table covers the main CLIs only; the exhaustive choices for every command are in the reference appendix. All model arrays are float64. Pascal consumer GPUs have low FP64 throughput, and no speed claim is made for either backend.

Select a GPU with CUDA_VISIBLE_DEVICES=<index or UUID>; the runner uses device 0 of the visible set. On shared machines, check that the device is idle first.

Dense GPU event: places the whole small case on the device after a memory preflight.

Windowed GPU run: first sets a hard limit on the CuPy default memory pool, equal to 0.85 × the device's *current* free memory (or a stricter existing limit). It does this before uploading anything and restores the previous limit afterwards.

Out-of-memory: inside that limit, it becomes an explicit resource refusal. The failing physical attempt is not adopted or checkpointed; earlier accepted checkpoints remain valid. Initial-store or diagnostic artifacts may already exist. A final successful output directory is not produced for the failed run.

Memory plan: the pre-run plan is known to undercount. Passing it does not mean the run fits.

# 6. Plot 1 end-to-end

## 6.1 Import and compile the corrected case

"$BIN/maple-syrup-plot1-import" \
  --recipe "$SYRUP/cases/plot1/recipe.yaml" \
  --mahleran-root "$MAHLERAN" \
  --expected-maple-root "$MAPLE" \
  --output-dir "$RUNS/cases/plot1_maple39"

--mahleran-root overrides the recipe's /home/okin/MAHLERAN default without editing the recipe. The importer:

1. stages and hashes every input it reads;

2. writes the MAPLE case package;

3. calls MAPLE's real compile_case, then reloads and checks the compiled state;

4. writes syrup/plot1_binding.json last.

The binding records the case identity, the compiled-artifact hash and the MAPLE package digest. On failure it writes syrup/FAILED.json (with traceback) and no binding; delete that directory and use a new path. --no-compile writes the audited package without compiling, which is useful only for inspection.

Exit codes: 0 ok; 1 import or MAPLE rejection; 2 MAPLE dependency refusal.

Every later runner re-verifies the case before using it:

bound file hashes;

MAPLE's load_compiled_case re-hash;

a fresh audit from the recipe and MAHLERAN inputs;

the bound MAPLE digest.

If you change MAPLE, re-import (Section 6.5).

## 6.2 Run the conservative event: dense bed, CPU and GPU

cd "$WORK"
CUDA_VISIBLE_DEVICES='' "$BIN/maple-syrup-event" \
  --case-dir "$RUNS/cases/plot1_maple39" \
  --output-dir "$RUNS/event_cpu" \
  --expected-maple-root "$MAPLE" \
  --max-end-s 5400

CUDA_VISIBLE_DEVICES=0 "$GBIN/maple-syrup-event" \
  --backend cupy --implementation cuda \
  --case-dir "$RUNS/cases/plot1_maple39" \
  --output-dir "$RUNS/event_gpu" \
  --expected-maple-root "$MAPLE" \
  --max-end-s 5400

--max-end-s is a horizon, not a target. After the rain stops, the event keeps running until the completion gates hold: --hold-s 60, --max-depth-m 1e-8, --surface-volume-m3 1e-6, --max-unit-discharge-m2-s 1e-10, --outlet-discharge-m3-s 1e-9, and the mobile-mass significance threshold. It then makes a forced final terrain commit and the explicit dry reset (Section 7).

If the horizon is reached first, the run fails and writes no output directory. It prints IncompleteEventError: maximum event duration reached; water and sediment were not reset; last accepted checkpoint: <path> and exits with status 1. That is not completion.

On success, the JSON summary is printed to stdout and saved as completion_summary.json. Defaults are listed in the reference appendix; the main ones are --checkpoint-every-s 600, --report-every-s 60, --max-dt-s 1, characteristic transport with 32 bins, and bisection with 40 iterations.

The 5400 s horizon in these examples is a chosen limit, not a newly verified completion time for this source pair. If the strict mobile-load gate prevents drying, a new run can explicitly select --deposit-mobile-at-flow-end: once all hydraulic gates have held for the quiet period, it deposits the remaining finite mobile mass through MAPLE. This is a model assumption, not a relaxed conservation tolerance; retain the flag in every continuation of that run.

## 6.3 Run the resident-window bed (CPU and GPU)

CUDA_VISIBLE_DEVICES='' "$BIN/maple-syrup-windowed" \
  --case-kind plot1 --backend numpy \
  --case-dir "$RUNS/cases/plot1_maple39" \
  --store-dir "$RUNS/stores/win_cpu" \
  --output-dir "$RUNS/win_cpu" \
  --max-end-s 5400

CUDA_VISIBLE_DEVICES=0 "$GBIN/maple-syrup-windowed" \
  --case-kind plot1 --backend cupy \
  --case-dir "$RUNS/cases/plot1_maple39" \
  --store-dir "$RUNS/stores/win_gpu" \
  --output-dir "$RUNS/win_gpu" \
  --max-end-s 5400

The windowed runner keeps a few top voxels per column resident (--window-voxels 4). The deep voxels go to an immutable, versioned MAPLE host store under --store-dir. Final outputs and checkpoints contain resident arrays plus references to store versions, so they are not standalone archives. Keep each store for as long as any output or checkpoint that references it must stay readable or resumable. Old versions are never rewritten. A failed attempt may leave an unreferenced version, but it never damages referenced ones.

Resource controls, all integers in bytes or records:

--max-host-bytes (40 GiB): bounds the host assembly planning estimate and the default checkpoint payload allowance, not the process's operating-system RSS;

--max-store-bytes (256 GiB): bounds the initial deep store's logical size, not the total disk space of every later retained version;

--max-journal-records (1,000,000);

--phase-max-nnz (1,000,000; refused before allocation, mass is never truncated);

--max-checkpoint-bytes (defaults to --max-host-bytes and may only tighten it).

The windowed runner disables avalanching for this water-only scope and records the original setting.

--reuse-initial restores the persisted initial window assembly from an existing --store-dir instead of re-importing it. The source's provenance is recorded. The examples above give each run its own fresh store, which is the simplest safe choice.

Full vs overlay deep storage.

--deep-chunk-encoding full is the default.

overlay stores a raw base plus one cumulative, exact final-value overlay per chunk. No tolerance-based dropping and no chains are used.

Overlay is accepted only when the imported MAPLE package digest is a qualified typed source. For this guide that is af54626… (MAPLE 39bcf79); one older MAPLE source is also accepted for historical reproduction. Otherwise it is refused before any case verification or store creation.

Overlay is also refused for cases with more voxels than MAPLE's OVERLAY_MAX_VOXELS, and for dense beds.

--deep-overlay-max-bytes applies only with overlay.

The encoding is part of the bed-storage identity. A run must resume with the same encoding, and a v2 (overlay) store cannot continue under full.

## 6.4 Pause and resume (same source, same backend, same controls)

CASE=$RUNS/cases/plot1_maple39

# Part 1: run to an original boundary and pause (checkpoint written at the pause)
CUDA_VISIBLE_DEVICES='' "$BIN/maple-syrup-event" --case-dir "$CASE" \
  --output-dir "$RUNS/event_pr_part1" --expected-maple-root "$MAPLE" \
  --max-end-s 5400 --pause-at-s 1200

CKPT=$("$BIN/python" -c 'import json,sys; print(json.load(open(sys.argv[1]))["last_checkpoint"])' \
       "$RUNS/event_pr_part1/completion_summary.json")
echo "$CKPT"     # .../event_pr_part1_checkpoints/step_NNNNNNNNN

# Part 2: NEW output directory, identical controls and --max-end-s, same interpreter/backend
CUDA_VISIBLE_DEVICES='' "$BIN/maple-syrup-event" --case-dir "$CASE" \
  --output-dir "$RUNS/event_pr_part2" --expected-maple-root "$MAPLE" \
  --max-end-s 5400 --resume "$CKPT"

A paused run exits 0 with "status": "paused". --pause-at-s must be a future *original* boundary (rain knots or report times, multiples of 60 s by default) before the horizon. Otherwise the run is refused with pause time must be a future original boundary before the duration limit.

For windowed runs, use the same pattern and give both parts the same `--store-dir`. A resume restores the initial assembly from that store automatically.

Resume is strict. The checkpoint's recorded identity must equal the current one exactly. That identity covers:

the SYRUP source digest and the resolved MAPLE package digest;

the case identity and binding, and the MAPLE configuration;

the MAHLERAN XML and rainfall hashes;

the sediment and column parameters;

the solver and transport controls, the completion policy and the graph (pit/nodata) policy;

--max-end-s, the report cadence and the row cap;

the backend (plus --cuda-mode on CUDA);

for windowed runs, the window, caps, initial deep manifest and deep encoding.

CPU checkpoints do not resume on CuPy, and GPU checkpoints do not resume on CPU. --max-checkpoint-bytes, --checkpoint-every-s and --pause-at-s are resource or scheduling controls and are excluded from the identity. During a run, the SYRUP source, MAPLE source, MAHLERAN XML and rainfall file are re-hashed at every checkpoint, and any change aborts the run. Do not git pull, edit or reinstall while a run is active.

The resumed run carries the checkpoint's accumulated result and origin, so its hydrograph continues from the event origin. Check t_s[0] before concatenating anything yourself.

## 6.5 When MAPLE changes: --allow-maple-source-change

Normal runs should not use this flag: re-import the case with the current pin instead. The flag only acknowledges, at case verification, that the MAPLE digest recorded at import differs from the MAPLE imported now. The difference is recorded in the summary (backend.maple_source_changed_since_import).

It does not override any of these:

checkpoint/restart identity (a resume still refuses a checkpoint with a different recorded MAPLE);

the overlay/deferred-publication source guards;

source-bound windowed store and initial-assembly bindings;

the within-run source re-hashing.

Old cases, outputs, checkpoints and source locks from earlier MAPLE or SYRUP versions keep their original identities. Normal run/restart commands do not migrate or rebind them. The separate maintainer-only initial_reimport utility can reuse a genuinely initial Chastre assembly under an exact reviewed producer-to-consumer transition. It requires an independently reviewed artifact and unchanged original operands; its own unreviewed proposal is refused. It does not convert a wet checkpoint or authorize an ordinary user to bypass source guards.

# 7. Event semantics you need to interpret results

Completion and dry reset. "status": "complete" means every quiet gate held for --hold-s after the rain ended, the forced final commit succeeded and the post-commit checks passed. The dry reset then removes the remaining surface water and soil water as an external, reported removal (reset_accounting in the summary). Mobile sediment is never erased; it must have been deposited first.

`--deposit-mobile-at-flow-end` (off by default) is an explicit end-of-event assumption that the bed dries again. Once all hydraulic gates hold, MAPLE deposits the remaining finite mobile load, which is reported separately. It waives only the mobile-mass criterion and is not a mass tolerance.

Exact dry handoff. Only a complete run writes handoff/, the dry-state bundle. A completed handoff cannot be resumed as a wet event.

Terminal pit storage.

- *Dense runner:* opt in with --allow-pit-storage. Cells with no strictly lower D4 neighbour then deposit their mobile load through MAPLE and keep their water. --allow-flat-storage extends this to flat sinks.   - *Windowed runner:* pit storage is on automatically for rfid/chastre (flat storage too for Chastre) and unavailable for plot1.   - *No overflow.* Pits have no spill, overtopping or backwater. --terminal-storage-water lets an event complete with water ponded only in terminal-storage cells, and the reset removes it. Large Chastre terminal depths are therefore not credible pond predictions.

Routing refresh. Hydraulics reroute only when the largest per-cell |committed surface − held routing surface| reaches 0.00075 m. Time, reports, checkpoints and ordinary accounting commits never reroute by themselves.

Excluded from these runners: running a wind event (the dry handoff bed is the hand-over point; scheduling is MASY's job), direct dry-cell splash, vegetation growth, full ET, nutrients and pit overflow.

Pond/splash API (no installed CLI). maple_syrup.pond_event is a separate, opt-in Python API. It couples local-inertial pond hydraulics with actual overflow/export at outlet faces, dry rain splash, multi-face characteristic transport and the native MAPLE bed. It supports PondEventControl(bed_storage="windowed", ...) and its own checkpoints (pond_checkpoint). It is not reachable from any console command; see docs/splash_and_overflow/pond_event.md and docs/pond_windowed/README.md. Local inertia is an alternative hydraulic model, not MAHLERAN method 5/6.

# 8. Other workflows

## 8.1 Water-only and diagnostic runners

C=$RUNS/cases/plot1_maple39
"$BIN/maple-syrup-columns"  --case-dir "$C" --max-dt-s 1 --output-dir "$RUNS/columns"
"$BIN/maple-syrup-storm"    --case-dir "$C" --output-dir "$RUNS/storm_cpu"
CUDA_VISIBLE_DEVICES=0 "$GBIN/maple-syrup-storm" --case-dir "$C" --backend cupy --implementation cuda \
  --output-dir "$RUNS/storm_gpu"
"$BIN/maple-syrup-sediment" --case-dir "$C" --output-dir "$RUNS/sediment_window"

Columns are no-routing infiltration diagnostics. They write column_summary.json and final_columns.npz.

Storm is a water-only storm and recession on frozen terrain with no sediment and no reset. It writes storm_summary.json, final_water.npz, hydrograph.npz and hydrograph.csv.

Sediment runs water and wet sediment on the actual bed up to `--end-s` (default 5400 s). It has no stop criterion, reset or restart, so it is not a complete storm. It writes sediment_summary.json, final_state.npz (including initial_committed_elevation_m), hydrograph.npz, hydrograph.csv and maple_final_snapshot.npz, which comes from MAPLE's own save_state_snapshot.

## 8.2 Benchmark and legacy replay: needs reference artifacts you probably do not have

"$BIN/maple-syrup-benchmark" --case-dir "$C" --output-dir "$RUNS/legacy_replay" \
  --applied-rainfall "$REF/applied_rainfall.csv" \
  --fortran-ledger "$REF/syrup_sediment_ledger.dat" \
  --reference-run "$REF/mahleran_deterministic_ksat_run"

The default --transport-scheme legacy dispatches to maple_syrup.legacy_experiment (maple-syrup-legacy runs the same thing). It requires artifacts from an executed, instrumented MAHLERAN reference run:

the logged applied-rainfall CSV;

the Fortran sediment ledger;

a reference-run directory with execution.json and Output/.

The default values of those flags (outputs/phase7/..., outputs/phase7b/...) are relative paths to git-ignored local results. A fresh clone does not contain them, so this workflow is not turnkey. $REF above stands for wherever you hold such artifacts. If they are missing, the run stops with legacy replay failed: actual Fortran reference ledger and reference run execution.json are required.

Controls the replay cannot honour are refused with exit status 2 (legacy replay refused: ...):

--max-dt-s other than 1;

--backend cupy;

--phase-bins;

Courant or substep options.

Outputs: legacy_ledger.npz and legacy_summary.json.

--transport-scheme characteristic (or upwind) selects the explicit conservative benchmark instead: actual MAPLE exchange on frozen hydraulic geometry with the reference applied rainfall. It still requires --applied-rainfall, and has no completion, dry reset, restart, wind or GPU.

maple_syrup.legacy_driver (CPU/Numba) and maple_syrup.legacy_gpu_driver (resident GPU) support more cases: --case-kind plot1|rfid|chastre. For Plot 1 they use the parsed case rainfall unless you pass --applied-rainfall, and they need no Fortran ledger. They write legacy_ledger.npz, legacy_summary.json and, with --snapshot-times, legacy_snapshots.npz (Section 10). A failure leaves <out>.FAILED/FAILED.json. Both drivers use frozen terrain, unlimited supply and an explicit clipping source.

## 8.3 Optional case kinds: RFID and Chastre

These cases need extra external inputs and use distinct import commands, not maple-syrup-plot1-import.

RFID_2014 (0.1 m, 106 × 60 including the ring, nodata −9999). You must supply two inputs that are not in Git:

$MAHLERAN/Input/RFID_2014/, with XML version 1.2.0;

$NATIVE_CAPTURE, the syrup_hydro_steps.txt written by an executed, instrumented native MAHLERAN run of RFID_2014.

The recipe pins the capture's SHA-256 (capture_expected_sha256). A different capture file is refused. The recipe's hydrology values, including its fixed conductivity, come from an audit of that native run. Treat them as values for this one benchmark, not as turnkey defaults for other sites.

export NATIVE_CAPTURE=/abs/path/to/your/syrup_hydro_steps.txt   # user-supplied artifact
"$BIN/python" -m maple_syrup.rfid_case \
  --recipe "$SYRUP/cases/rfid/recipe.yaml" \
  --mahleran-root "$MAHLERAN" --expected-maple-root "$MAPLE" \
  --applied-forcing-capture "$NATIVE_CAPTURE" \
  --output-dir "$RUNS/cases/rfid_maple39"

Chastre is a 1 m tiled case (1393 × 1604 interior, about 33 GB of persisted bed). It needs the separately obtained terrain DTM_Chastre_PD4way.asc, which the recipe pins by SHA-256, and a completed RFID case. The recipe holds absolute /home/okin/... paths, and both --plan-only and generation read them as written. Relative paths would resolve against your current directory, not the recipe's directory.

Copy the recipe and set both paths to absolute locations you own:

terrain.path: your copy of the DTM;

source_rfid.case_dir: your RFID case.

Keep terrain.sha256 and every other value unchanged. --plan-only always reads source_rfid.case_dir from the recipe and ignores --source-case-dir. Editing the recipe therefore keeps the plan and the generation consistent, and you can then omit --source-case-dir.

mkdir -p "$WORK/recipes" && cp "$SYRUP/cases/chastre/recipe.yaml" "$WORK/recipes/chastre.yaml"
# edit $WORK/recipes/chastre.yaml:
#   terrain.path:        /abs/path/to/DTM_Chastre_PD4way.asc
#   source_rfid.case_dir: <the value of $RUNS>/cases/rfid_maple39   (write it out in full)
cd "$WORK"
"$BIN/python" -m maple_syrup.chastre_case --recipe "$WORK/recipes/chastre.yaml" \
  --output-dir "$RUNS/cases/chastre_maple39" --plan-only      # writes nothing; exit 3 if disk too small
"$BIN/python" -m maple_syrup.chastre_case --recipe "$WORK/recipes/chastre.yaml" \
  --output-dir "$RUNS/cases/chastre_maple39" --expected-maple-root "$MAPLE"
"$BIN/python" -m maple_syrup.chastre_case --verify-case "$RUNS/cases/chastre_maple39" --hash-only-tiles

The case records the copied recipe's absolute path and hash, so keep that copy unchanged after generation.

Run the conservative event on these cases with the windowed runner. You must pass the case's declared nodata value; it is never inferred, and a mismatch is refused.

CUDA_VISIBLE_DEVICES='' "$BIN/maple-syrup-windowed" --case-kind rfid --backend numpy \
  --case-dir "$RUNS/cases/rfid_maple39" --store-dir "$RUNS/stores/rfid_cpu" \
  --output-dir "$RUNS/rfid_cpu" --nodata-value -9999 --terminal-storage-water

For Chastre, earlier large runs used --phase-max-nnz 4000000 --max-journal-records 4000000. The largest documented run used a separately prepared *rotated* Chastre case. It ran through the Python API run_complete_windowed(...) with API-only deferred publication (deep_publication='boundary'), not through this CLI, and it stopped incomplete at its 10,000 s horizon. A full large storm is not known to fit or complete.

# 9. Outputs: files, units, orientation

Read every NPZ with allow_pickle=False. All arrays are float64 unless stated.

Orientation.

Grid arrays are (ny, nx) with row 0 = south. Plot with origin="lower".

Legacy ASCII grids are north-first; the importer crops the one-cell ring and reverses the rows.

Sidecar legacy_full_* fields in plot1_fields.npz keep the ring (62 × 22) and are also south-first.

Class axes are last, in canonical class order (c1 = finest phi_1 … c6 = coarsest).

Event and windowed outputs (`<output>/`):

| File / key | Meaning | Units |
| --- | --- | --- |
| completion_summary.json | status, identity, budgets, closures, reset accounting, routing counts, timings, outputs SHA-256 map | — |
| final_state.npz: depth_m, soil_water_m | for a complete run, the state after the dry reset (the removed water is in reset_accounting); for a paused run, the wet state at the pause | m |
| pre_reset_depth_m, pre_reset_soil_water_m | physical state at the end of the accepted event, before any reset; use these for final water maps | m |
| discharge_m2_s | unit discharge | m² s⁻¹ |
| committed_elevation_m | MAPLE committed surface (MAPLE datum: legacy DEM + datum_offset_m) | m |
| active_mass_kg, available_mass_kg, bound_mass_kg, mobile_mass_kg | per cell and class | kg |
| voxel_mass_kg (dense) | full voxel column (ny, nx, nz, nc) | kg |
| window_kg (windowed) | resident window voxels (ny, nx, W, nc) | kg |
| deep_by_class_kg, deep_total_kg, deep_room_kg (windowed) | per-cell totals over the deep voxels below the window: mass by class, total mass, unfilled capacity. The voxel-by-voxel deep contents live in the store | kg |
| base (windowed) | (ny, nx) int32 logical voxel index of window slot 0, i.e. the first resident level | dimensionless index, not kg |
| terminal_deposition_kg | final settling deposit (zeros if not complete) | kg |
| sediment_velocity_m_s | transport velocity memory per cell and class | m s⁻¹ |
| phase_fraction / phase_csr_fraction | share of the cell/class mobile mass held in each position bin | dimensionless |
| phase_position_m / phase_csr_position_m | representative distance from the cell's upstream face | m |
| phase_csr_row_ptr, phase_csr_bin, phase_csr_cell_of_slot | CSR structure | integer indices |
| hydrograph.npz | one row per boundary; water columns t_s, cumulative_rain_m3, cumulative_intake_m3, cumulative_saturation_return_m3, cumulative_drainage_m3, cumulative_export_m3, surface_storage_m3, soil_storage_m3, outlet_discharge_m3_s, max_depth_m, … ; sediment columns prefixed sed_ (sed_mobile_kg, sed_cumulative_export_kg, sed_export_rate_kg_s, per-class sed_cumulative_export_c1_kg …) | m³, m³ s⁻¹, kg, kg s⁻¹ |
| exchange_history.npz | opt-in cumulative per-cell/class MAPLE ledger touch counts and gross activity | counts, kg |

Interpreting the hydrograph.

Columns named cumulative_* are running totals since the event origin. Get interval volumes or masses as differences of consecutive rows.

outlet_discharge_m3_s is the instantaneous water discharge at the end of the last accepted step before the row.

sed_export_rate_kg_s (and per-class sed_export_rate_c*_kg_s) is the actual export of that last accepted step divided by its Δt. It is a last-step average, not an average over the report interval.

For a mean over a report interval, divide the change in a cumulative column by the change in t_s, e.g. np.diff(h["sed_cumulative_export_kg"]) / np.diff(h["t_s"]). Never multiply either of the rate columns by the report interval to estimate a volume or yield.

Rows sit at report times (60 s) plus rain knots, so spacing can be irregular.

The final row includes terminal settling and the final commit, before the external reset.

Bed change. final_state.npz stores only the final committed surface. For a Dense Plot 1 event, compute the change against the initial committed surface of the same compiled case, in the same datum frame:

import numpy as np
from maple.case_tools.compilers.case_compiler import load_compiled_case
from maple.core.types.topographic_commit import committed_elevation_m

case_dir = "/abs/path/runs/cases/plot1_maple39"     # same case the run used
run_dir = "/abs/path/runs/event_cpu"
case = load_compiled_case(case_dir)
z0 = np.asarray(committed_elevation_m(case.committed_topography))
with np.load(f"{run_dir}/final_state.npz", allow_pickle=False) as z:
    z1 = z["committed_elevation_m"]
dz_m = z1 - z0          # + deposition, - erosion; same MAPLE datum, so the offset cancels

Run this in the MAPLE environment. maple-syrup-sediment already saves initial_committed_elevation_m alongside the final surface.

For a windowed event, use the actual initial assembly saved in that run's store. Windowed initialization can regroup the Dense baseline for its exact height-packet arithmetic; substituting a separately loaded Dense case can show a small apparent change even before erosion. The saved initial baseline uses the same native committed-elevation definition:

from pathlib import Path
import numpy as np

store = Path("/abs/path/runs/stores/win_cpu")
run = Path("/abs/path/runs/win_cpu")
with np.load(store / "initial_baseline.npz", allow_pickle=False) as initial:
    z0 = initial["reference_elevation_m"] + initial["elevation_offset_m"]
with np.load(run / "final_state.npz", allow_pickle=False) as final:
    dz_m = final["committed_elevation_m"] - z0
np.savez("/abs/path/runs/figures/windowed_change.npz", dz_m=dz_m)

Keep the matching store, initial binding and run summary. On a paused run the committed surface can lag the physical bed's pending changes; a completed event forces final accounting. A height-change map of committed surfaces must be labeled accordingly.

exchange_history.npz and the per-cell cumulative_* maps are accumulations over the whole event. They are not time-resolved frames, and you cannot reconstruct a movie from them.

# 10. Figures and movies

What exists. No committed console runner writes general evolving-bed spatial movie frames. The explicit saved map sequences are:

experimental_experiment --snapshot-times-s t1,t2,… writes snapshots.npz: water-only fixed-terrain maps keyed t<time>_<field> (for example t600_depth_m, t600_soil_water_m, t600_velocity_m_s, and face fluxes t600_qx_m2_s/t600_qy_m2_s for local inertia). The snapshot times are inserted as solver boundaries.

legacy_driver / legacy_gpu_driver --snapshot-times (at most 16 whole seconds) writes legacy_snapshots.npz with stacked t_s, depth_m and mobile_kg. These are fixed-bed legacy maps. Check their shapes with np.load(..., allow_pickle=False) before plotting.

A movie of a conservative evolving bed would need explicit capture code written against the event API and checked against the source. There is currently no CLI option for it.

Checkpoints (<output>_checkpoints/step_*) hold complete wet states at the checkpoint cadence (600 s by default). A custom script could decode them through the checkpoint API with the run's own context and identity. That is developer work, not a movie command, and the frames would be at checkpoint times only.

## 10.1 The companion plotting example

$MANUALS/render_outputs.py is a small companion example; it is not an installed SYRUP command. It needs only NumPy and Matplotlib. It never runs the model, never touches a GPU or checkpoint, and never invents maps that were not saved. Its subcommands are:

inspect: list the keys, shapes and dtypes in an NPZ without loading the payloads;

map: draw one saved 2-D map, or one class (--class-index, zero-based) or the class sum (--sum-classes) of a 3-D (row, column, class) map;

hydrograph: plot one column of hydrograph.npz against t_s;

frames: render saved snapshots to PNG frames with one fixed colour scale.

Maps are drawn with row 0 at the bottom (south). With --dx-m the axes are in metres. The helper refuses to overwrite an existing output file or frames directory.

The helper assumes cell-centred maps. Staggered qx_m2_s and qy_m2_s need their own face coordinates and should not be passed to its cell-map command. With stacked legacy snapshots it holds one full selected field stack in memory; use small cases or a limited number of snapshots when memory is tight.

H="$MANUALS/render_outputs.py"
mkdir -p "$RUNS/figures"
"$BIN/python" "$H" inspect "$RUNS/event_cpu/final_state.npz"
"$BIN/python" "$H" map "$RUNS/event_cpu/final_state.npz" --key pre_reset_depth_m \
  --dx-m 0.5 --label "Depth before dry reset (m)" --output "$RUNS/figures/event_cpu_depth.png"
"$BIN/python" "$H" map "$RUNS/event_cpu/final_state.npz" --key mobile_mass_kg --sum-classes \
  --dx-m 0.5 --label "Mobile sediment, all classes (kg)" --output "$RUNS/figures/event_cpu_mobile.png"
"$BIN/python" "$H" hydrograph "$RUNS/event_cpu/hydrograph.npz" --key outlet_discharge_m3_s \
  --label "Outlet discharge, end of last step (m³/s)" --output "$RUNS/figures/event_cpu_q.png"
"$BIN/python" "$H" hydrograph "$RUNS/event_cpu/hydrograph.npz" --key sed_cumulative_export_kg \
  --label "Cumulative sediment export (kg)" --output "$RUNS/figures/event_cpu_sed_export.png"

For a complete run, map pre_reset_depth_m rather than depth_m, which is zero after the dry reset. The default --label is Depth (m), so always set a label that matches the key. To map bed change, compute dz_m as in Section 9 and save it to a new NPZ (np.savez(path, dz_m=dz_m)). Then use map --key dz_m --cmap RdBu_r with an explicit symmetric --vmin/--vmax and label Committed height change (m).

## 10.2 Making a water-depth movie from experimental snapshots

"$BIN/python" -m maple_syrup.experimental_experiment \
  --case-dir "$RUNS/cases/plot1_maple39" --output-dir "$RUNS/exp_explicit" \
  --solver explicit --backend numpy --implementation numba \
  --max-dt-s 1 --end-s 600 --expected-maple-root "$MAPLE" \
  --snapshot-times-s 60,120,180,240,300,360,420,480,540,600

"$BIN/python" "$H" frames "$RUNS/exp_explicit/snapshots.npz" --field depth_m \
  --dx-m 0.5 --label "Water depth (m)" --frames-dir "$RUNS/frames"

ffmpeg -framerate 4 -i "$RUNS/frames/frame_%05d.png" \
  -c:v libx264 -pix_fmt yuv420p "$RUNS/exp_explicit_depth.mp4"

Snapshot times must lie in (start, end]: 0 is refused, and the run starts at 0 s. Each requested time becomes an exact solver boundary. This is a water-only, fixed-terrain experimental solver, not the conservative event and not equivalent to the legacy hydraulics.

## 10.3 Legacy-replay snapshots

"$BIN/python" -m maple_syrup.legacy_driver --case-kind plot1 \
  --case "$RUNS/cases/plot1_maple39" --output "$RUNS/legacy_plot1" \
  --end-s 600 --snapshot-times 60,120,180,240,300,360,420,480,540,600
"$BIN/python" "$H" frames "$RUNS/legacy_plot1/legacy_snapshots.npz" --field mobile_kg --sum-classes \
  --dx-m 0.5 --label "Mobile sediment, all classes (kg)" --frames-dir "$RUNS/frames_legacy_mobile"

legacy_snapshots.npz stacks t_s (n), depth_m (n, ny, nx) and mobile_kg (n, ny, nx, nc). Select a class with --class-index 0, or sum all classes with --sum-classes. These are frozen-terrain legacy maps with unlimited supply. They are not evolving-bed results.

## 10.4 Reading movies correctly

Colour scale. frames scans all frames first and uses one fixed scale, which it prints. Pass --vmin/--vmax to choose a physical range yourself, and use the same range when comparing movies.

Time. Frame rate is playback speed only. The helper writes frame_times_s.json and warns when snapshot spacing is unequal; real time maps linearly onto video time only for evenly spaced snapshots. Solver steps are adaptive in the experimental runner, so each frame is labelled with its actual time.

Pixel size. The helper saves fixed-size figures (792 × 624 px at its frame resolution), which suits -pix_fmt yuv420p. If a custom Matplotlib configuration changes the saved size (for example savefig.bbox: tight), add -vf "scale=trunc(iw/2)*2:trunc(ih/2)*2" to the FFmpeg command.

No invented frames. There are no spatial flux or bed-change frames to render, because no runner saves them over time.

# 11. Scope limits

Supply and availability. MAPLE's native water exchange caps requested pickup by the currently available mass, using exactly the same rule as wind erosion. The bound share is not released automatically. It then caps pickup per class at the active-layer holdings before refilling from the voxel bed. The corrected Plot 1 case starts with every class 100% available. Prescribed pavement and vegetation affect the water laws independently of that native availability split.

Wind. A complete event hands over a dry native bed; MASY owns the sequential wind/water scheduling and calls each model. No SYRUP CLI runs or schedules a wind event.

Storage-admission checks. maple_syrup.storage_admission (admit_public, admit_paired) provides opt-in caller checks for memory admission of windowed exchanges. Existing runners do not call it automatically.

Qualification status. The 39bcf79 pairing is qualified for the storage integration (docs/maple39_shared_storage_qualification.md). That does not qualify:

- alternating wind–water events;   - large-catchment storms or whole-Chastre memory fit;   - universal CPU/GPU bitwise equality.

Historical status. README sections and dated status blocks in older docs are historical. Where they disagree with current source and with docs/maple39_shared_storage_qualification.md and docs/pond_windowed/, follow the source and those docs.

# 12. Troubleshooting

| Message (abridged) | Cause | Fix |
| --- | --- | --- |
| import maple failed | wrong interpreter | use $BIN/$GBIN (or the Section 1.6 PYTHONPATH) |
| expected MAPLE source root X, but the imported maple resolves to source root Y | a different MAPLE (e.g. the shared live checkout) | check MAPLE_SYRUP_EXPECTED_MAPLE_ROOT, the venv, PYTHONPATH |
| imported maple at … is not inside the editable install root | another copy shadows it on sys.path | unset stray PYTHONPATH; leave directories containing maple* |
| MAHLERAN XML not found, XML version … != expected '1.2.3', input_folder … not the recipe's | MAHLERAN inputs missing or different | fix --mahleran-root and the inputs (Section 3) |
| calib.dat exists | legacy calibration not supported | remove it from your input copy, then re-import |
| refusing to write into existing path, output and checkpoint directories must be new | reused path | choose new --output-dir (and its _checkpoints sibling) |
| refusing to write inside the … tree | output under MAPLE/MAHLERAN/recipe/SYRUP package | write under $RUNS |
| …/syrup/FAILED.json exists: the import failed | case directory from a failed import | re-import into a new directory |
| MAPLE source digest … differs from the one bound at import | case imported under another MAPLE | re-import with the current pin (Section 6.5) |
| Numba requested but unavailable; no implicit fallback | missing Numba | install [windowed] / [numba] extras |
| PrefixPacketError: decode_height … needs the compiled Numba kernel | windowed CPU run without Numba | install MAPLE [windowed] |
| backend 'cupy' with implementation 'numba' is refused | invalid pairing | --backend cupy --implementation cuda |
| backend 'cupy' is refused: … / backend unavailable | no CuPy or no visible device | use .venv-cupy; check CUDA_VISIBLE_DEVICES, driver |
| Failed to find CUDA headers | CuPy without [ctk] | reinstall [windowed,gpu] |
| allow_flat_storage extends allow_pit_storage and needs it; terminal_storage_water needs an opt-in pit-storage graph | dense pit options missing | add --allow-pit-storage (dense); not available for windowed plot1 |
| masked case requires explicit nodata_value / nodata_value … differs from the verified case grid | windowed rfid/chastre | pass --nodata-value -9999 (the declared value) |
| deep_chunk_encoding='overlay' is supported only with an accepted MAPLE source | MAPLE digest not qualified | use the 39bcf79 pin or full |
| pause time must be a future original boundary … | pause not on a report/rain boundary | pick a multiple of the report cadence before the horizon |
| IncompleteEventError: maximum event duration reached; … last accepted checkpoint: … | horizon reached before completion | not a result; inspect or resume only with the same --max-end-s (identity), or start a new run with a longer horizon |
| checkpoint schema, case/source/config identity or payload set mismatch; restart stopping or numerical controls changed; restart duration or forcing changed | resume with different source/backend/controls | resume with exactly the original sources, backend and controls |
| SYRUP source changed during the event / MAPLE source changed … | files edited or reinstalled mid-run | do not touch the trees during runs |
| resource refusal: CuPy device memory request exceeded the enforced pool hard limit … | does not fit the device | earlier checkpoints stay valid; free the device or use CPU |
| resource refusal: the resident-window planning estimate alone exceeds 85% … | estimate already too large | smaller case or larger device |
| legacy replay refused: … (exit 2) / actual Fortran reference ledger … required | unsupported legacy control / missing reference artifacts | Section 8.2 |

Exit codes.

0: success, including paused runs.

1: run, import or validation failure.

2: dependency refusal, usage error (argparse) or legacy refusal.

3: chastre_case --plan-only disk refusal.

The dense runners print errors to stderr. The windowed runner prints progress and errors to stdout.

# 13. Reproducibility checklist

Record these for every study:

1. Exact commits and cleanliness. Capture git -C "$MAPLE" rev-parse HEAD, git -C "$SYRUP" rev-parse HEAD and git status --porcelain for both. HEAD alone does not identify uncommitted edits; the digests do.

2. Probe report. Save it (--output), with the MAPLE package digest af54626…, source_root, Python and package versions. Also save pip freeze from each environment.

3. Case provenance.

- syrup/plot1_binding.json: case identity, artifact hash, MAPLE digest, SYRUP provenance, environment.    - syrup/plot1_import_report.json: recipe and XML hashes, staged-source hashes, resolved legacy settings, datum offset.    - MAPLE's provenance.yaml.

4. Run summary (completion_summary.json):

- identity: syrup_source_sha256, maple_source_sha256, case_identity_sha256, mahleran_xml_sha256, rainfall_sha256, sediment_parameters, column_parameters, control, completion_policy, graph_policy, backend;    - backend.maple_source_changed_since_import;    - outputs, the SHA-256 of every output file.

5. Retention. Keep every checkpoint directory you might resume from, and every windowed --store-dir that is referenced. Treat them as immutable.

6. Dependency pins and bundles (advanced). dependencies/maple_shared_storage/README.md (reconstruction recipe, fixed manifest SHA-256 877826de…), qualification.lock.json, docs/cw_e2_dependency_delivery.md (bundle export/verify/materialize/env/check-runtime) and docs/maple39_shared_storage_qualification.md. Bundles certify integrity, not authenticity, and do not carry third-party packages or deep-store data.

# Appendix A. Complete CLI option reference

This reference was generated from the argument parsers in committed SYRUP d05d8c6, using the same captured help and parser actions. Every installed command and the seven module-only utilities listed in the guide are included. Aliases appear together. A False Boolean flag is off until supplied; True/False here is a switch state, not a value to type. A required option must be supplied even if its parser default is null. “Not supplied” frequently resolves from the case or another option; important resolutions are described in the guide and the meaning column. Every command also accepts -h / --help, which prints help and exits. Quoting string choices is optional in ordinary Bash usage.

The generic benchmark has two interfaces: its default execution delegates to the legacy parser, while generic --help shows the explicitly conservative branch. This table distinguishes them. Parser acceptance does not override documented scientific restrictions or validation. In particular, a parsed unsupported legacy option is deliberately refused, and method-5 terminal storage does not implement spill.

The tables reproduce parser defaults, not a claim that every combination is compatible. Resource limits do not become scientific tolerances. Keep all output paths new and use matching source/backend/configuration for restart.

## A1. maple-syrup-probe

Dependency and backend probe.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --backend | "numpy"; choices: numpy, cupy | Array backend; select the matching implementation where the runner exposes both controls. |
| --device-id | 0; int | Visible-device index, after any CUDA_VISIBLE_DEVICES remapping. Default 0. |
| --expected-maple-root | Not supplied (runtime resolution) | Refuse to run unless the imported MAPLE resolves to this source root (default: $MAPLE_SYRUP_EXPECTED_MAPLE_ROOT when set). |
| --output | Not supplied (runtime resolution) | Also write the JSON report to this NEW file. |
| --include-file-manifest | false | Include per-file SHA-256 entries in the provenance record. |

## A2. maple-syrup-plot1-import

Corrected Plot1 importer; compilation is on unless --no-compile is passed.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --recipe | Required | Authored recipe, e.g. cases/plot1/recipe.yaml |
| --output-dir | Required | NEW directory for the case package. |
| --mahleran-root | Not supplied (runtime resolution) | Override the recipe's MAHLERAN root (read-only). |
| --expected-maple-root | Not supplied (runtime resolution) | Refuse any other MAPLE source root. |
| --no-compile | false | Write the audited case package but do not call MAPLE's compiler. |

## A3. maple-syrup-columns

Independent infiltration columns; no routing or sediment.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --case-dir | Required | Verified Phase 2 case, e.g. outputs/plot1 |
| --max-dt-s | Required; float | Largest step (s); knots always split. |
| --output-dir | Required | NEW directory for the summary and final grids. |
| --backend | "numpy"; choices: numpy, cupy | Array backend; select the matching implementation where the runner exposes both controls. |
| --mahleran-root | Not supplied (runtime resolution) | Override the recipe's MAHLERAN root (read-only). |
| --expected-maple-root | Not supplied (runtime resolution) | Refuse any other MAPLE source root. |
| --allow-maple-source-change | false | Run even if MAPLE's source digest differs from the import's (recorded). |

## A4. maple-syrup-storm

Water-only directed storm to a specified end time; not event completion.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --case-dir | Required | Verified Phase 2 case, e.g. outputs/plot1 |
| --output-dir | Required | NEW directory for summary, grids and hydrograph. |
| --max-dt-s | 1.0; float | Largest step (s); knots always split. |
| --end-s | Not supplied (runtime resolution); float | Simulated end (s); default the legacy stormlength (5400 s for Plot 1). |
| --implementation | "numba"; choices: array, numba, cuda | Hydrology implementation (default numba; missing Numba is an error, no fallback). 'cuda' is the explicit water-only prepared CUDA hydrology: it REQUIRES --backend cupy and a CUDA device and never needs Numba (no fallback). |
| --backend | "numpy"; choices: numpy, cupy | Array backend; select the matching implementation where the runner exposes both controls. |
| --root-solver | "bisection"; choices: bisection, newton | Cell-equation root solver (default bisection, unchanged). 'newton' is the explicit safeguarded Newton (CPU array/numba, or CUDA with --implementation cuda). |
| --newton-max-iterations | 50; int | Newton pass cap per cell, 1..1000 (only meaningful with --root-solver newton). |
| --report-every-s | 60.0; float | Hydrograph row cadence (s). |
| --min-dt-s | 0.0009765625; float | Retry floor in seconds; a forcing/report boundary may itself require a shorter slice. |
| --max-retries | 10; int | Maximum recoverable step retries before a refusal. |
| --max-steps | 10000000; int | Guard on accepted steps; exceeding it is not physical completion. |
| --mahleran-root | Not supplied (runtime resolution) | Override the recipe's MAHLERAN root (read-only). |
| --expected-maple-root | Not supplied (runtime resolution) | Refuse any other MAPLE source root. |
| --allow-maple-source-change | false | Run even if MAPLE's source digest differs from the import's (recorded). |

## A5. maple-syrup-sediment

Conservative sediment time window; no complete-event reset or restart.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --case-dir | Required | Verified Phase 2 case, e.g. outputs/plot1 |
| --output-dir | Required | NEW directory for summary, grids, hydrograph, snapshot. |
| --max-dt-s | 1.0; float | Largest step (s); knots always split. |
| --end-s | Not supplied (runtime resolution); float | Simulated end (s); default the legacy stormlength (5400 s for Plot 1). |
| --implementation | "numba"; choices: array, numba, cuda | Hydraulic sweep implementation (default numba; missing Numba is an error, no fallback; 'cuda' is the explicit prepared CUDA hydrology and requires --backend cupy). |
| --backend | "numpy"; choices: numpy, cupy | numpy (array/numba hydraulics) or cupy (ONLY with --implementation cuda: whole small bed on the device, memory preflight first); other pairings are refused, no fallback. Select the device with CUDA_VISIBLE_DEVICES before launching. |
| --root-solver | "bisection"; choices: bisection, newton | Cell-equation root solver of the hydraulic sweep (default bisection). |
| --cuda-mode | "auto"; choices: auto, fused, split | CUDA hydrology launch structure (auto/fused/split; cupy backend only). |
| --report-every-s | 60.0; float | Hydrograph row cadence (s). |
| --min-dt-s | 0.0009765625; float | Retry floor in seconds; a forcing/report boundary may itself require a shorter slice. |
| --max-retries | 10; int | Maximum recoverable step retries before a refusal. |
| --max-steps | 10000000; int | Guard on accepted steps; exceeding it is not physical completion. |
| --sediment-courant-max | 0.5; float | Sediment Courant cap per substep (<= 0.5 for characteristic, <= 1 for upwind). |
| --max-transport-substeps | 64; int | Cap on internal sediment substeps per physical step. Exceeding it rejects the attempt; it never truncates a path or deletes mass. |
| --transport-scheme | "characteristic"; choices: characteristic, upwind | Lateral transport: characteristic (Phase 7b phase bins, default) or the Phase 5 upwind operator kept as an explicit comparison. |
| --phase-bins | 32; int | Position bins per cell/class for the characteristic scheme (1..128; 32 is a candidate default, not an accepted convergence). |
| --transport-implementation | "auto"; choices: auto, array, numba, cuda | Characteristic kernel: auto (follow --implementation), array or numba (dense storage); numba or cuda (csr storage); no fallback. |
| --phase-storage | "dense"; choices: dense, csr | Phase storage of the characteristic scheme: dense (default, the accepted (ny,nx,nc,B) arrays) or csr (the accepted sparse nonempty-bin state; same physics and bins). |
| --phase-max-nnz | Not supplied (runtime resolution); int | csr only: predeclared capacity (stored bins) refused BEFORE allocation when exceeded; never truncates mass. Required by the CUDA operator for large cases. |
| --no-commit | false | Do not evaluate MAPLE commit triggers per step. |
| --no-final-commit | false | Do not force a final commit at end_s. |
| --mahleran-root | Not supplied (runtime resolution) | Override the recipe's MAHLERAN root (read-only). |
| --expected-maple-root | Not supplied (runtime resolution) | Refuse any other MAPLE source root. |
| --allow-maple-source-change | false | Run even if MAPLE's source digest differs from the import's (recorded). |

## A6. maple-syrup-event

Complete conservative evolving-bed Plot1 event, Dense storage.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --case-dir | Required; Path | Verified imported case package directory. |
| --output-dir | Required; Path | New output directory. Existing output paths and protected input/source locations are refused. |
| --resume | Not supplied (runtime resolution); Path | Existing source/config/backend-bound wet checkpoint bundle, not a completed dry handoff. |
| --checkpoint-dir | Not supplied (runtime resolution); Path | Checkpoint destination; default is the output path plus _checkpoints. |
| --max-checkpoint-bytes | 1073741824; int | bound on the summed uncompressed payload bytes of the --resume bundle (positive integer, default 1 GiB); a resource control, not part of the run identity |
| --checkpoint-every-s | 600.0; float | Checkpoint scheduling interval in model seconds; this does not change hydraulic routing. |
| --pause-at-s | Not supplied (runtime resolution); float | Pause at a future original report/forcing boundary before the duration limit; preserve wet checkpoint. |
| --max-end-s | Not supplied (runtime resolution); float | Maximum event duration in seconds. Reaching it without physical completion is a refusal, not drying. |
| --max-dt-s | 1.0; float | Largest physical timestep in seconds; forcing/report boundaries and accepted retries can make it shorter. |
| --implementation | "numba"; choices: array, numba, cuda | hydraulic sweep: numba (default) or array on the host; cuda requires --backend cupy |
| --backend | "numpy"; choices: numpy, cupy | numpy (array/numba) or cupy (ONLY with --implementation cuda); other pairings refused |
| --bisection-iterations | 40; int | explicit root iteration budget; accuracy and conservation bounds unchanged |
| --root-solver | "bisection"; choices: bisection, newton | Bisection or safeguarded Newton for the same directed hydraulic equation. |
| --cuda-mode | "auto"; choices: auto, fused, split | CUDA hydrology launch structure (cupy backend only) |
| --allow-maple-source-change | false | accept an explicitly recorded isolated MAPLE source difference from the import binding |
| --allow-pit-storage | false | opt-in: active strict sinks become terminal-storage cells (deposit, keep water, no export) |
| --allow-flat-storage | false | opt-in with --allow-pit-storage: flat sinks are terminal storage too |
| --allow-masked-nodata | false | opt-in: inactive / ring cells may hold --nodata-value and are never receivers |
| --nodata-value | Not supplied (runtime resolution); float | the DEM nodata sentinel for --allow-masked-nodata |
| --terminal-storage-water | false | completion option: flow cessation with water ponded ONLY in terminal-storage cells (removed and accounted by the external dry reset); needs --allow-pit-storage |
| --deposit-mobile-at-flow-end | false | CW-B51 completion option: explicit dry-again assumption; after rain ended and every hydraulic quiet gate held for --hold-s, the remaining finite mobile load is deposited through MAPLE (reported separately); not a mass tolerance; default off |
| --record-exchange-history | false | CW-D2: record the actual per-cell/class MAPLE ledger touch counts and gross activity through every commit (exchange_history.npz); diagnostic evidence only, default off |
| --report-every-s | 60.0; float | Reporting cadence in model seconds. Forcing knots may add rows; it does not write spatial movie frames. |
| --hold-s | 60.0; float | Continuous accepted hydraulic quiet period required before completion, in seconds. |
| --max-depth-m | 1e-08; float | Maximum moving surface depth allowed by the stopping policy (m); physical quiet gate. |
| --surface-volume-m3 | 1e-06; float | Maximum moving surface-water volume allowed by the stopping policy (m³). |
| --max-unit-discharge-m2-s | 1e-10; float | Physical stopping threshold on unit-width discharge (m²/s). |
| --outlet-discharge-m3-s | 1e-09; float | Physical stopping threshold on summed outlet discharge (m³/s). |
| --mobile-per-cell-class-kg | Not supplied (runtime resolution); float | physical stopping significance; default actual MAPLE mass_resolution_kg, not a conservation tolerance |
| --max-steps | 10000000; int | Guard on accepted steps; exceeding it is not physical completion. |
| --max-report-rows | 100000; int | Guard on the number of report rows planned/stored. |
| --transport-scheme | "characteristic"; choices: characteristic, upwind | characteristic (Phase 7b phase bins, default) or the Phase 5 upwind comparison |
| --phase-bins | 32; int | position bins per cell/class for the characteristic scheme (1..128) |
| --transport-implementation | "auto"; choices: auto, array, numba, cuda | characteristic kernel: auto (follow --implementation), array or numba (dense storage), numba or cuda (csr storage); no fallback |
| --phase-storage | "dense"; choices: dense, csr | phase storage of the characteristic scheme: dense (default) or csr (sparse nonempty-bin state, same physics and bins; CSR checkpoints and final arrays) |
| --phase-max-nnz | Not supplied (runtime resolution); int | csr only: predeclared capacity of stored bins, refused before allocation when exceeded; never truncates mass |
| --sediment-courant-max | 0.5; float | Maximum virtual-sediment-speed × substep duration / cell width; characteristic transport requires a value no greater than 0.5. |
| --mahleran-root | Not supplied (runtime resolution); Path | External read-only legacy input tree; overrides the recipe root where supported. |
| --expected-maple-root | Not supplied (runtime resolution); Path | Refuse an imported MAPLE that does not resolve to this exact source root. |

## A7. maple-syrup-windowed

Complete conservative resident-window event; selects Numba for numpy, CUDA for cupy. Phase storage is CSR. This CLI has no --implementation, --max-dt-s or --expected-maple-root flag; use its actual options and the expected-root environment variable.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --case-dir | Required | Verified imported case package directory. |
| --output-dir | Required | New output directory. Existing output paths and protected input/source locations are refused. |
| --store-dir | Required | Native immutable deep-store destination. Keep referenced versions for checkpoint/output use. |
| --case-kind | Required; choices: plot1, rfid, chastre | Choose the matching case importer/verification path. |
| --backend | "numpy"; choices: numpy, cupy | Array backend; select the matching implementation where the runner exposes both controls. |
| --bisection-iterations | 40; int | Root iteration budget; exact default and case-specific resolution described above. |
| --root-solver | "bisection"; choices: bisection, newton | Bisection or safeguarded Newton for the same directed hydraulic equation. |
| --max-end-s | Not supplied (runtime resolution); float | Maximum event duration in seconds. Reaching it without physical completion is a refusal, not drying. |
| --pause-at-s | Not supplied (runtime resolution); float | Pause at a future original report/forcing boundary before the duration limit; preserve wet checkpoint. |
| --reuse-initial | false | Restore persisted native initial window assembly instead of creating it again. |
| --resume | Not supplied (runtime resolution) | Existing source/config/backend-bound wet checkpoint bundle, not a completed dry handoff. |
| --checkpoint-every-s | 600.0; float | Checkpoint scheduling interval in model seconds; this does not change hydraulic routing. |
| --phase-max-nnz | 1000000; int | Explicit capacity for sparse phase bins; exceeding it refuses, rather than truncates mass. |
| --max-journal-records | 1000000; int | Maximum pending native deep-journal records, not a license to discard changes. |
| --window-voxels | 4; int | Requested number of resident subactive voxel levels; native geometry rules still apply. |
| --max-host-bytes | 42949672960; int | Host assembly/checkpoint planning allowance in bytes; not an enforced process RSS cap or whole-storm fit guarantee. |
| --max-checkpoint-bytes | Not supplied (runtime resolution); int | resume bound on summed uncompressed bundle bytes; default = --max-host-bytes, may only tighten it |
| --max-store-bytes | 274877906944; int | Initial native deep-store logical-byte allowance; not a quota covering every retained later version. |
| --deep-chunk-encoding | "full"; choices: full, overlay | CW-B108 deep-chunk storage: full (default) or exact overlay (accepted typed MAPLE source only) |
| --deep-overlay-max-bytes | Not supplied (runtime resolution); int | overlay only: explicit per-publication overlay byte cap forwarded to MAPLE (default: MAPLE rule) |
| --nodata-value | Not supplied (runtime resolution); float | Explicit DEM nodata sentinel; must agree with the case, never inferred as a physical elevation. |
| --allow-flat-storage | Not supplied (runtime resolution) | Default unset: enabled automatically for chastre; optional for rfid; refused for plot1, whose CLI path has no pit storage. These are terminal sinks, not overflow hydraulics. |
| --terminal-storage-water | false | Allow event quiet gates to exclude recognized terminal-storage water; retain it in water accounting until explicit reset. Requires pit storage: rfid/chastre only; refused for plot1 in this CLI. |
| --deposit-mobile-at-flow-end | false | Explicit modeling option: settle remaining finite mobile load through MAPLE after hydraulic quiet gates hold; not a looser conservation tolerance. |
| --record-exchange-history | false | Write cumulative native per-cell/class touch/gross-exchange evidence, not temporal spatial frames. |
| --allow-maple-source-change | false | Record deliberate MAPLE drift from import binding. Does not waive checkpoint, storage or within-run source guards. |

## A8. maple-syrup-legacy; default maple-syrup-benchmark

Legacy replay, CPU Plot1. The generic benchmark dispatches here unless characteristic/upwind is explicitly selected. Options labeled unsupported are parsed to give a clear refusal; they do not enable the corresponding feature.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --output / --output-dir | Required; Path | NEW output directory (legacy_ledger.npz, legacy_summary.json) |
| --case / --case-dir | "outputs/plot1" | Verified imported case package directory. |
| --applied-rainfall | "outputs/phase7/mahleran_reference_audit/applied_rainfall.csv" | Legacy-applied rainfall capture CSV; external benchmark input, not a bundled default dataset. |
| --depth-time-level | "previous"; choices: previous, current | legacy evaluates the laws with the PREVIOUS step's depth d(1) and the new velocity |
| --end-s | Not supplied (runtime resolution); float | End of the requested time window in seconds; None resolves from the case/driver. This flag alone does not imply completion. |
| --implementation | "numba"; choices: numba, array | SYRUP water-routing implementation. numba (default) requires compiled water AND compiled legacy sediment kernels, with no silent Python fallback. array is a declared slow diagnostic of the water sweep only. |
| --physics-implementation | Not supplied (runtime resolution); choices: numba, array | wet physical-law evaluation. Default: the same as --implementation. numba = fused compiled CPU kernel on a prepared frozen-composition context (explicit error if Numba is missing, no silent fallback). array = the NumPy sediment_physics_step reference (declared slower; may be combined with compiled hydrology and legacy transport for controlled comparisons). |
| --hydrology-implementation | Not supplied (runtime resolution); choices: prepared, reference | rainfall/infiltration/routing step. Default: prepared with --implementation numba, reference with --implementation array. prepared = fused compiled CPU kernels on a prepared fixed-terrain context (same equations, bisection and checks; requires --implementation numba and host NumPy, explicit error if Numba is missing, no silent fallback). reference = the NumPy/CuPy-reference storm.coupled_step (with the numba or array sweep per --implementation); declared slower, kept for controlled comparisons. |
| --fortran-ledger | "outputs/phase7b/mahleran_ledger_run_v3/Output/syrup_sediment_ledger.dat"; Path | External Fortran sediment ledger used by the narrow Plot1 legacy comparison. |
| --reference-run / --reference-run-dir | "outputs/phase7/mahleran_deterministic_ksat_run"; Path | External native reference run directory; it must contain the expected comparison artifacts. |
| --allow-maple-source-change | false | Record deliberate MAPLE drift from import binding. Does not waive checkpoint, storage or within-run source guards. |
| --transport-scheme | "legacy"; choices: legacy | Transport model selection. The narrow legacy replay accepts only legacy; choose an explicitly conservative runner for characteristic/upwind. |
| --max-dt-s | Not supplied (runtime resolution); float | only 1 is supported |
| --report-every-s | Not supplied (runtime resolution); float | only 1 is supported |
| --backend | Not supplied (runtime resolution) | only numpy is supported (no GPU) |
| --phase-bins | Not supplied (runtime resolution) | unsupported for legacy: multi-bin characteristic transport (deferred to Phase 7g; use --transport-scheme characteristic) |
| --transport-implementation | Not supplied (runtime resolution) | unsupported for legacy: characteristic kernel selection |
| --sediment-courant-max | Not supplied (runtime resolution) | unsupported for legacy: sediment Courant control (the legacy replay has none) |
| --max-transport-substeps | Not supplied (runtime resolution) | unsupported for legacy: transport substeps (the legacy replay has none) |
| --min-dt-s | Not supplied (runtime resolution) | unsupported for legacy: adaptive time stepping (the legacy replay is fixed dt = 1 s) |
| --max-retries | Not supplied (runtime resolution) | unsupported for legacy: step retries (the legacy program has no retry) |
| --max-steps | Not supplied (runtime resolution) | unsupported for legacy: step limit |
| --max-report-rows | Not supplied (runtime resolution) | unsupported for legacy: hydrograph report rows |
| --expected-ksat-mm-s | Not supplied (runtime resolution) | unsupported for legacy: conductivity check (the matched benchmark only) |
| --mahleran-root | Not supplied (runtime resolution) | unsupported for legacy: MAHLERAN root override (the matched benchmark only) |
| --expected-maple-root | Not supplied (runtime resolution) | unsupported for legacy: MAPLE root pin (the matched benchmark only) |

## A9. maple-syrup-benchmark --transport-scheme characteristic|upwind

Explicit conservative fixed-geometry comparison only. The parser tables below describe this explicit branch; default legacy execution uses the preceding table. --applied-rainfall is required here, not for generic --help.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --case-dir | Required | Verified Phase 2 case, e.g. outputs/plot1 |
| --output-dir | Required | NEW directory for summary, grids and hydrographs. |
| --applied-rainfall | Required | CSV of the reference run's logged applied rainfall (audit_mahleran.py). |
| --reference-run | Not supplied (runtime resolution) | Deterministic-conductivity MAHLERAN run directory to bind (execution.json, Output/). |
| --max-dt-s | 1.0; float | Largest step (s): 1, 0.5 or 0.25 for the study. |
| --end-s | Not supplied (runtime resolution); float | Window end (s); default the legacy 5400 s. |
| --implementation | "numba"; choices: array, numba | Hydraulic sweep: numba (default, CPU compiled; missing Numba is an error) or array. |
| --backend | "numpy"; choices: numpy, cupy | Only numpy is accepted; cupy is refused explicitly (no GPU claim). |
| --report-every-s | 1.0; float | Hydrograph cadence (s); 1 s for matching. |
| --min-dt-s | 0.0009765625; float | Retry floor in seconds; a forcing/report boundary may itself require a shorter slice. |
| --max-retries | 10; int | Maximum recoverable step retries before a refusal. |
| --max-steps | 10000000; int | Guard on accepted steps; exceeding it is not physical completion. |
| --max-report-rows | 100000; int | Guard on the number of report rows planned/stored. |
| --sediment-courant-max | 0.5; float | Sediment Courant cap per substep (<= 0.5 for characteristic, <= 1 for upwind). |
| --max-transport-substeps | 64; int | Cap on internal sediment substeps per physical step. Exceeding it rejects the attempt; it never truncates a path or deletes mass. |
| --transport-scheme | "legacy"; choices: legacy, characteristic, upwind | legacy (DEFAULT): MAHLERAN legacy replay, dispatched to maple_syrup.legacy_experiment. characteristic: Phase 7b conservative phase-bin model (explicit). upwind: Phase 5 operator kept as an explicit comparison. |
| --phase-bins | 32; int | Position bins per cell/class for the characteristic scheme (1..128; 32 is a candidate default, not an accepted convergence). |
| --transport-implementation | "auto"; choices: auto, array, numba, cuda | Characteristic kernel: auto (follow --implementation), array or numba; no fallback. |
| --expected-ksat-mm-s | 0.00025; float | Assert the matched benchmark conductivity (mm/s) on every active cell; this checks the selected case instead of changing its Ksat. |
| --mahleran-root | Not supplied (runtime resolution) | Override the recipe's MAHLERAN root (read-only). |
| --expected-maple-root | Not supplied (runtime resolution) | Refuse any other MAPLE source root. |
| --allow-maple-source-change | false | Run even if MAPLE's source digest differs from the import's (recorded). |

## A10. python -m maple_syrup.experimental_experiment

Water-only explicit/local-inertial experiments; snapshots possible. NumPy backend implementation is named numpy, not array.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --case-dir | Required | Verified Phase 2 case, e.g. outputs/plot1 |
| --output-dir | Required | NEW directory for summary, grids, hydrograph and snapshots. |
| --solver | Required; choices: explicit, local_inertial | Choose explicit D4 kinematic wave or two-dimensional local inertia. These are different water equations, not alternative roots of method 5. |
| --backend | Not supplied (runtime resolution); choices: numpy, cupy | array namespace: numpy (default) or cupy (needs CuPy + device; never Numba; no fallback) |
| --implementation | Not supplied (runtime resolution); choices: numpy, numba, cuda | solver form: numpy = NumPy reference, numba = compiled CPU form (needs numpy backend and Numba), cuda = CUDA form (needs cupy). Omitted: numpy for --backend numpy, cuda for --backend cupy. Invalid pairs are refused before the case is read; there is never a silent fallback |
| --max-dt-s | 1.0; float | Largest physical timestep in seconds; forcing/report boundaries and accepted retries can make it shorter. |
| --end-s | Not supplied (runtime resolution); float | default: the legacy stormlength (5400 s for Plot 1) |
| --report-every-s | 60.0; float | Reporting cadence in model seconds. Forcing knots may add rows; it does not write spatial movie frames. |
| --cfl-max | 0.5; float | CFL bound in (0, 0.5] |
| --limiter | "off"; choices: off, donor | 'donor' = documented conservative positivity limiter (local inertia only); default off |
| --snapshot-times-s | Not supplied (runtime resolution) | comma-separated times for synchronous snapshot maps Times must be in (start, end]; zero is refused. |
| --min-dt-s | 0.0009765625; float | Retry floor in seconds; a forcing/report boundary may itself require a shorter slice. |
| --max-retries | 10; int | Maximum recoverable step retries before a refusal. |
| --max-steps | 10000000; int | Guard on accepted steps; exceeding it is not physical completion. |
| --mahleran-root | Not supplied (runtime resolution) | Override the recipe's MAHLERAN root (read-only). |
| --expected-maple-root | Not supplied (runtime resolution) | Refuse any other MAPLE source root. |
| --allow-maple-source-change | false | Run even if MAPLE's source digest differs from the import's (recorded). |

## A11. python -m maple_syrup.rfid_case

RFID-specific import; the shipped recipe requires an external native-applied forcing capture.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --recipe | Required | Authored case recipe; referenced external data must be present and match its hashes. |
| --output-dir | Required | NEW directory for the case package. |
| --applied-forcing-capture | Not supplied (runtime resolution) | syrup_hydro_steps.txt of the executed native run |
| --mahleran-root | Not supplied (runtime resolution) | External read-only legacy input tree; overrides the recipe root where supported. |
| --expected-maple-root | Not supplied (runtime resolution) | Refuse an imported MAPLE that does not resolve to this exact source root. |
| --no-compile | false | Audit/write inputs without native compilation; this is not a completed runnable case. |

## A12. python -m maple_syrup.chastre_case

Tiled Chastre plan/generate/verify. Required combinations depend on the chosen mode, although argparse shows them as optional.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --recipe | Not supplied (runtime resolution) | cases/chastre/recipe.yaml (plan / generate) |
| --output-dir | Not supplied (runtime resolution) | NEW case directory (plan / generate) |
| --plan-only | false | print bands, estimates and free disk; write nothing |
| --verify-case | Not supplied (runtime resolution) | verify a completed case directory |
| --tile-rows | Not supplied (runtime resolution); int | Number of rows per compiled Chastre tile; when absent, use the authored recipe. |
| --source-case-dir | Not supplied (runtime resolution) | Override the RFID source case for Chastre generation. Plan-only reads source_rfid.case_dir from the recipe; set that path too. |
| --expected-maple-root | Not supplied (runtime resolution) | Refuse an imported MAPLE that does not resolve to this exact source root. |
| --allow-maple-source-change | false | Record deliberate MAPLE drift from import binding. Does not waive checkpoint, storage or within-run source guards. |
| --hash-only-tiles | false | verify: stream-hash tiles but reload only tile 0 |

## A13. python -m maple_syrup.legacy_driver

Extended CPU legacy replay for Plot1/RFID/Chastre; fixed bed, clipping-source diagnostics.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --case-kind | Required; choices: plot1, rfid, chastre | Choose the matching case importer/verification path. |
| --case / --case-dir | Required; Path | verified imported case directory |
| --output / --output-dir | Required; Path | NEW output directory |
| --end-s | Not supplied (runtime resolution); float | default: the case storm length; whole seconds |
| --snapshot-times | "" | comma-separated model seconds (at most 16); depth and mobile maps |
| --warmup-s | 0.0; float | whole model seconds run first on throwaway state in-process |
| --progress-every-s | 60.0; float | stderr progress cadence in model seconds; 0 = off |
| --root-solver | "bisection"; choices: bisection, newton | Bisection or safeguarded Newton for the same directed hydraulic equation. |
| --bisection-iterations | Not supplied (runtime resolution); int | default 40 (Plot 1) / 64 (RFID, Chastre) |
| --newton-max-iterations | Not supplied (runtime resolution); int | Safeguarded Newton iteration budget; when not supplied, use the case/driver resolution (the root implementation defaults to 50). |
| --depth-time-level | "previous"; choices: previous, current, post_infiltration | wet-law depth: previous (default, accepted), current (new depth), post_infiltration (the original's d(1)) |
| --source-order | "index"; choices: index, legacy | Legacy walk source processing order: index uses south-first flat indices; legacy uses north-first rows then west to east. CUDA refuses legacy order. |
| --legacy-depos-erase | false | reproduce the order-dependent erasure of deposition at wet no-rain cells (needs --source-order legacy) |
| --applied-rainfall | Not supplied (runtime resolution); Path | Plot 1 only: applied-rainfall CSV (as legacy_experiment) |
| --allow-maple-source-change | false | Record deliberate MAPLE drift from import binding. Does not waive checkpoint, storage or within-run source guards. |
| --hash-only-tile-verify | false | Chastre: reload tile 0 only during verification |
| --hash-tiles-after-run | false | Chastre: stream-hash every persisted tile after the run (reads the whole ~33 GB bed state once) |
| --max-memory-gib | 32.0; float | refuse if the estimated engine buffers exceed this |
| --allow-python-kernels | false | TESTS ONLY: pure-Python kernels (very slow) |

## A14. python -m maple_syrup.legacy_gpu_driver

Extended CuPy legacy replay. Inherited CPU diagnostic flags may be explicitly refused: source-order legacy, legacy-depos-erase and allow-python-kernels are unsupported on CUDA.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --case-kind | Required; choices: plot1, rfid, chastre | Choose the matching case importer/verification path. |
| --case / --case-dir | Required; Path | verified imported case directory |
| --output / --output-dir | Required; Path | NEW output directory |
| --end-s | Not supplied (runtime resolution); float | default: the case storm length; whole seconds |
| --snapshot-times | "" | comma-separated model seconds (at most 16); depth and mobile maps |
| --warmup-s | 0.0; float | whole model seconds run first on throwaway state in-process |
| --progress-every-s | 60.0; float | stderr progress cadence in model seconds; 0 = off |
| --root-solver | "bisection"; choices: bisection, newton | Bisection or safeguarded Newton for the same directed hydraulic equation. |
| --bisection-iterations | Not supplied (runtime resolution); int | default 40 (Plot 1) / 64 (RFID, Chastre) |
| --newton-max-iterations | Not supplied (runtime resolution); int | Safeguarded Newton iteration budget; when not supplied, use the case/driver resolution (the root implementation defaults to 50). |
| --depth-time-level | "previous"; choices: previous, current, post_infiltration | wet-law depth: previous (default, accepted), current (new depth), post_infiltration (the original's d(1)) |
| --source-order | "index"; choices: index, legacy | Legacy walk source processing order: index uses south-first flat indices; legacy uses north-first rows then west to east. CUDA refuses legacy order. |
| --legacy-depos-erase | false | reproduce the order-dependent erasure of deposition at wet no-rain cells (needs --source-order legacy) |
| --applied-rainfall | Not supplied (runtime resolution); Path | Plot 1 only: applied-rainfall CSV (as legacy_experiment) |
| --allow-maple-source-change | false | Record deliberate MAPLE drift from import binding. Does not waive checkpoint, storage or within-run source guards. |
| --hash-only-tile-verify | false | Chastre: reload tile 0 only during verification |
| --hash-tiles-after-run | false | Chastre: stream-hash every persisted tile after the run (reads the whole ~33 GB bed state once) |
| --max-memory-gib | 32.0; float | refuse if the estimated engine buffers exceed this |
| --allow-python-kernels | false | TESTS ONLY: pure-Python kernels (very slow) |
| --check-every-steps | 60; int | read the device flags/ledger rows every N steps (and at the end) |
| --cuda-mode | "auto"; choices: auto, fused, split | hydrology launch structure |
| --cn-mode | "auto"; choices: auto, level, block | Crank-Nicolson launch structure |
| --gpu-memory-gib | Not supplied (runtime resolution); float | refuse if the estimated device memory exceeds this |
| --water-accounting | "fused"; choices: separate, fused | per-step water series and cumulative maps: 'fused' = one device launch per step, 'separate' = the B1/B2 reference (3 scalar copies + 4 adds); identical arithmetic, device-only either way (default fused: provisional) |
| --record-strategy | "compact"; choices: compact, all | walk record values: 'compact' = only classes with a positive composition fraction in an active cell (static eligibility, exactly the same science), 'all' = every class (the B1 reference) |

## A15. python -m maple_syrup.dependency_bundle

Advanced dependency source/metadata delivery utility; does not run physics. A materialized bundle carries native MAPLE source and truthful metadata, not the environment dependencies.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |

## python -m maple_syrup.dependency_bundle export

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --project | Required | Exact native MAPLE project selected for export. |
| --distribution-metadata | Required | Genuine matching native MAPLE dist-info directory; fabricated or stale metadata is refused. |
| --origin | Required | Authored origin/provenance JSON required by the export schema. |
| --output | Required | New output path; file for the probe, directory for a run. |

## python -m maple_syrup.dependency_bundle verify

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| <bundle> | Required | Existing verified native dependency bundle directory. |

## python -m maple_syrup.dependency_bundle materialize

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| <bundle> | Required | Existing verified native dependency bundle directory. |
| --cache | Required | Destination content-addressed materialization cache. |

## python -m maple_syrup.dependency_bundle verify-materialized

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| <target> | Required | Existing materialized native source/metadata directory. |

## python -m maple_syrup.dependency_bundle env

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| <target> | Required | Existing materialized native source/metadata directory. |
| --syrup-src | Required | Selected SYRUP src directory to include in the printed runtime environment. |

## python -m maple_syrup.dependency_bundle check-runtime

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| <target> | Required | Existing materialized native source/metadata directory. |

## A16. python -m maple_syrup.initial_reimport

Maintainer-only propose/import of a Chastre generation-zero initial assembly under an exact independently reviewed source transition. A proposal is unreviewed and cannot be imported as-is. This is not wet checkpoint migration, a routine source upgrade, or permission to waive any source guard.

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |

## python -m maple_syrup.initial_reimport propose

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --store-dir | Required | Native immutable deep-store destination. Keep referenced versions for checkpoint/output use. |
| --producer-package-dir | Required | Exact original producer MAPLE package directory; its complete file manifest is checked. |
| --expected-maple-root | Not supplied (runtime resolution) | Refuse an imported MAPLE that does not resolve to this exact source root. |
| --out | Required | New path for an unreviewed source-transition proposal. The proposal itself is not authorization to import. |

## python -m maple_syrup.initial_reimport import

| Option / alias | Default, type and choices | Meaning / constraints |
| --- | --- | --- |
| --store-dir | Required | Native immutable deep-store destination. Keep referenced versions for checkpoint/output use. |
| --producer-package-dir | Required | Exact original producer MAPLE package directory; its complete file manifest is checked. |
| --expected-maple-root | Not supplied (runtime resolution) | Refuse an imported MAPLE that does not resolve to this exact source root. |
| --case-dir | Required | Verified imported case package directory. |
| --metadata-dir | Required | New directory for initial-assembly metadata; original operands and deep-store versions are preserved. |
| --transition | Required | Exact source-transition JSON with an independent review record and matching producer/consumer manifests. |
| --max-host-bytes | Required; int | Host assembly/checkpoint planning allowance in bytes; not an enforced process RSS cap or whole-storm fit guarantee. |
| --allow-case-maple-source-change | false | Acknowledge a Chastre case-verification source difference only. Does not relax initial-assembly source/transition checks. |

# Appendix B. References and verification scope

Versioned implementation and requirements: https://github.com/gregokin/SYRUP/tree/d05d8c6c66d9fd2969c007681916254e8d1c2c7c and https://github.com/gregokin/MAPLE/tree/39bcf79e1e67b6ed960f78a923ddfedf1d173e68 . The optional CUDA dependency instructions were cross-checked with the official CuPy 14.2 installation documentation: https://docs.cupy.dev/en/stable/install.html . NVIDIA drivers remain a separate host prerequisite.

Documentation checks captured actual --help for all nine console-command modules and the seven module-only utilities, compared every reference-table flag/default/choice with the pinned parser, checked shell/Python example syntax, and exercised the companion plotting example with small synthetic NPZ files and a real CPU FFmpeg encode. No new storm simulation, GPU computation, environment installation or full fresh-machine installation was performed for the manuals. Historical qualification results remain bound to their recorded source pairs; this document does not extend them.